SmartDrain – Complete Project Explanation

Project Overview

SmartDrain is an IoT-based Smart Flood Monitoring and Early Warning System designed to help communities prepare for flooding before it happens.

The project combines:

- An ESP32 microcontroller

- A water level sensor

- A rain sensor

- A cardboard city with a drainage system

- Firebase Realtime Database

- OpenWeather API

- A modern HTML website

The main goal is to monitor drainage conditions in real time and combine them with weather forecasts to predict possible flooding.

Components Used

Hardware

- 1 × ESP32 Development Board

- 1 × Water Level Sensor

- 1 × Rain Sensor

- Breadboard

- Jumper wires

- USB power supply

- Cardboard city with drainage channels

Optional

- Buzzer

- LEDs

- LCD/OLED Display

Software

- HTML

- CSS

- JavaScript

- Firebase Realtime Database

- OpenWeather API

- Arduino IDE

System Architecture

OpenWeather API

│

Weather Forecast Data

│

▼

SmartDrain Website

│

Reads Weather + Sensor Data

│

▼

Firebase Realtime Database

▲

│

Uploads Sensor Readings

▲

│

ESP32

/                 \

/                   \

Water Level Sensor       Rain Sensor

│

▼

Cardboard Drainage City

How Everything Works

Step 1 — The User Opens the Website

The user opens the SmartDrain website.

The website automatically:

- asks permission to access the user's location

- detects the current city

- stores the location temporarily

- prepares to fetch weather information

Step 2 — OpenWeather Provides Weather Data

Using the user's location, the website sends a request to the OpenWeather API.

The API returns information such as:

- Current temperature

- Weather condition

- Humidity

- Wind speed

- Pressure

- Rain probability

- Five-day forecast

- Hourly forecast

The website displays this information in a beautiful weather dashboard.

Step 3 — ESP32 Monitors the Drainage

The ESP32 is connected to the miniature drainage system.

Every few seconds it reads:

- Water level

- Rain sensor status

For example

Water Level = 76%

Rain Sensor = TRUE

Step 4 — ESP32 Uploads Data

Instead of sending data directly to the website,

the ESP32 uploads it to Firebase.

Example

Firebase

Water Level = 76%

Rain = TRUE

Overflow = FALSE

Time = 2:15 PM

Firebase now becomes the central communication hub.

Why Firebase?

Think of Firebase like a cloud mailbox.

Instead of

ESP32 ⇄ Website

you use

ESP32 → Firebase ← Website

The ESP32 writes data.

The website reads data.

They never communicate directly.

This makes the system faster, more reliable, and easier to expand.

Step 5 — Website Reads Firebase

The SmartDrain website continuously listens for updates from Firebase.

Whenever the ESP32 uploads new sensor values,

Firebase instantly sends the new data to the website.

No page refresh is needed.

The dashboard updates automatically.

Step 6 — Flood Risk Analysis

Now the website has two sources of information.

Source 1

OpenWeather

Example

Heavy Rain Expected

Rain Probability = 92%

Source 2

ESP32

Example

Water Level = 88%

The website combines both values.

Example

Rain Probability = 92%

Water Level = 88%

Result

Flood Risk = HIGH

Step 7 — Alert System

If the risk becomes dangerous,

the website immediately displays

🚨 Flood Warning

Heavy rain expected.

Drainage water level is high.

Please avoid flood-prone areas.

The warning is also saved inside Firebase.

Dashboard Features

The website displays

Weather

- Current Temperature

- Humidity

- Wind Speed

- Pressure

- Sunrise

- Sunset

- Hourly Forecast

- Five-Day Forecast

Drainage

- Water Level

- Rain Sensor

- Overflow Status

- Sensor Status

- Last Update

Flood Analysis

- Low Risk

- Medium Risk

- High Risk

- Critical Risk

Notifications

- Heavy Rain

- Overflow

- Storm Warning

- Sensor Offline

- Flood Alert

Statistics

- Total Alerts

- Flood Events

- Sensor Health

- API Status

- Firebase Status

How Flood Risk Is Calculated

The website follows simple rules.

Rule 1

Water Level < 40%

Rain Probability < 40%

↓

LOW RISK

Rule 2

Water Level = 60%

Rain Probability = 65%

↓

MEDIUM RISK

Rule 3

Water Level > 70%

Rain Probability > 70%

↓

HIGH RISK

Rule 4

Water Level > 90%

Rain Probability > 90%

↓

CRITICAL RISK

Real-Life Example

Imagine it starts raining.

The rain enters the cardboard drainage.

The water level rises.

The ESP32 measures

Water Level = 91%

At the same time,

OpenWeather predicts

Heavy Rain

95% Probability

The website combines both values.

Result

🚨 CRITICAL FLOOD RISK

The dashboard changes to red.

The emergency alert appears.

Everyone using the website immediately sees the warning.

Data Flow

Rain falls

│

▼

Cardboard drainage fills

│

▼

Water Level Sensor measures water

│

▼

ESP32 reads sensor

│

▼

ESP32 uploads data to Firebase

│

▼

Website receives live sensor data

│

▼

Website requests OpenWeather forecast

│

▼

Website compares

• Weather Forecast

• Water Level

│

▼

Calculates Flood Risk

│

▼

Displays Dashboard

│

▼

Shows Emergency Alerts

Why This Project Is Useful

This project demonstrates how IoT, cloud services, and weather forecasting can work together to provide early flood warnings.

Instead of waiting until flooding occurs, the system continuously monitors both the current drainage condition and the upcoming weather forecast. This allows users to receive warnings before a flood becomes dangerous, making the system more useful for protecting homes, schools, campuses, and communities.

In a real-world deployment, multiple ESP32 devices could be installed in different drainage locations, all sending data to the same Firebase database. The website would then provide a centralized dashboard showing the status of every monitored area, making the system scalable from a single demonstration model to an entire community.